(				*** NOTICE ***

These pads have been validated by the MOSIS System of USC/ISI. Any

modification of these pads by the user is done solely at the user's

risk.  Modified pads are not to be referred to as "MOSIS Pads" unless

prior agreement is obtained from USC/ISI.

MOSIS really intends that you use the complete (stuffed) digital or

analog pad frames without modification.  The individual pads and pad

frame subcells are supplied only as a courtesy to those who want to

build and verify their own custom pad frames.  These pads were not

designed to be interchangeable within the pad frame, rather, they were

optimized for power and size, and require intricate editing and

verification in different arrangements.);

TINYCHIP PADS SET

SCN (N-WELL) 2.0 MICRON

1. INTRODUCTION

This 2 micron SCN (N-well) TinyChip pads set was designed with the

MOSIS scalable CMOS design rules (Rev 6) at a lambda of 1.0 micron.

To use these pads, get the newest Magic technology file (dated on or

after than 1/19/93).  All pads are laid out on a lambda grid and are

designed for use with the 40PC22X22 Standard Frame.

2. CONTENTS OF THE DIGITAL PADS SET

* List

blank.CIF		balnk with only bonding metals

gnd.CIF			internal GND power supply - not connected to pad GND

vdd.CIF			internal Vdd power supply - not connected to pad Vdd

in.CIF			input pad - a derivative from I/O pad

io.CIF			input/output pad

out.CIF			output pad - a derivative from I/O pad

cg.CIF			GND Bottom Left corner pad

CG_r.CIF		GND Bottom Right corner pad

cv.CIF			Vdd Top Left corner pad

cv_r.CIF		Vdd Top Right corner pad

40pc22x22_stuffed.CIF   contains a complete io pad ring layout in size

2220x2250 um

* Description

The single most important pad in this pad set is the IO pad. From this

tri-state I/O pad we derived the IN pad and the OUT pad. A circuit diagram

for this tri-state I/O pad is given below. Connecting ENABLE to GND will

turn this I/O pad into an input pad. Connecting ENABLE to Vdd will result

an output pad.

ESD protection is done with

(1) Thick-Field Oxide transistor of size W/L= 600/3 microns,

(2) 150 ohms N_diffusion resistor/diode, and

(3) Tri-state output drivers as pair of diode clamps.

* Circuit Diagram

+-----+

+----| PAD |

|    +-----+

+-+       |

Vdd                         |         \

+---------+                     +-+       / 150 ohm

|         |                       |GND    \

|+--+         +--+|  ENABLE                   |

+--0||               ||0---------         Vdd     |

|   |+--+         +--+|                    |      |

|       |         |                    |+--+      |

|       +---------+-------------------0|| P       |

|       |         |                    |+--+      |

ENABLE  |   |+--+         +--+| ENABLEbar          |      |

---------|---||               ||0--+------          +------+

|   |+--+         +--+|   |                |      | IN_unbuffered

|       |         |       |            |+--+      |

|       +---------+-------|------------|| N     +---+

|       |         |       |            |+--+    |   |

OUT   |   |+--+         +--+|   |                |     \ /

------+---||               ||---+              GND      0

|+--+         +--+|                           | INbar

|         |                             +---+

+---------+                             |   |

GND                                   \ /

0

|

IN

* Size

These pads should fit into the standard frame 40PC22x22 with

mirroring.  They can also be used for other MOSIS standard frames. io,

in, out, Vblank, vdd, and gnd has none-well maximum bonding box (MBB)

of 200x210 microns each. The well MBB is 210x218 microns. The stuffed

frame has a non-well MBB of 2220x2250 microns. There are 9 pads on

each edge plus 4 corners used as Vdd and GND pads. The top edge has a

GND pad in the middle and its 2 corners are Vdd pads, while the bottom

edge has a Vdd pad in the middle and its 2 corners are GND pads. This

gives 3 Vdd and 3 GND pads total.

3. CONTENTS OF THE ANALOG PADS SET

* List

gndN.CIF		internal GND power supply - not connected to pad GND

vddN.CIF		internal Vdd power supply - not connected to pad Vdd

analogN.CIF             analog input/output pad

llN.CIF			GND Bottom Left corner pad

urN.CIF			Vdd Top Right corner pad

40pc22x22_stuffed_analog.CIF   contains a complete io pad ring layout in size

2220x2250 um

* Description

ESD protection is achieved by two lateral PNP parasitic transistor in

N-well

(1) reverse biased diode connected to Vdd when a positive ESD stress

is applied.

(2) PNP transistor from GND to input when a negative ESD stress is

applied.

4. SIMULATION and MEASUREMENT

Both pad set have been fabricated by MOSIS for verification, but only

pads in digital set are simulated and measured. Their result are

listed as follows:

* Delay

OUTPUT Driver Load (SIMULATED)

10pF   30pF   50pF

------------+------------------------

rise time   |  8ns   21ns   30ns

fall time   |  7ns   17ns   25ns

------------+------------------------

OUTPUT Driver Load (MEASURED)

no load  10pF    22pF     47pF

------------+--------------------------------

prop. delay |  8.6ns  10.8ns  13.0ns   18.5ns

(0 to 1)   |

------------+--------------------------------

INPUT Driver Load (SIMULATED)

load:		load:		load:

INbar/IN	INbar/IN	INbar/IN

0.5/1pF		1/2pF		2/4pF

prop/rise/fall	prop/rise/fall	prop/rise/fall

signal	signal	delay/time/time	delay/time/time	delay/time/time

------	------	---------------	---------------	---------------

PAD	INbar	1.1/3.9/2.0	1.6/5.5/3.0	2.2/8.5/4.5

INbar	IN	2.2/3.0/2.2	2.2/5.0/3.5	3.8/8.5/6.5

* DC Current Capability

Maximum current :: 11 mA per I/O pad with 50pF loading

5. Magic CIF input

In Magic, Berkeley's layout editor, use ":cif is lambda=1.0(nwell)"

to specify the appropriate CIF input style before using ":cif read

..." command.