Date: 8 December 1990 9:36:53

From: The MOSIS System <XMOSIS@CHEETA.ISI.EDU>(send requests to MOSIS@MOSIS.EDU)

Subject: Topic:  Library: STD_FRAMES, File: STD_FRAMES.DOC

To: evans@wiener.tisl.ukans.edu

Standard Project Frames

-----------------------

The MOSIS service offers standard frames as an alternative to custom

project frames where every project pad layout is unique.  A standard

project frame is a rectangular area of a fixed size and with a fixed

set of bonding pad locations around the periphery of the rectangle.

The circuitry in the remainder of the frame, the routing of pad

connections, and the choice of pad types at each of the fixed

locations are up to the user.  The use of standard frames has numerous

advantages over custom frames to both the MOSIS service and the user

community:

1. The  bonding of projects in standard frames is always the same for a

given frame; therefore, packaging is both faster and more reliable.

2. Designers using standard frames determine their own package pin-out.

3. The future MOSIS pad routing  service  will  support  automatic  pad

routing to the standard frame chosen.

4. The future MOSIS functional testing service will provide probe cards

for  the  standard  frames;  projects  in custom frames will have to

create their own private probe cards.

The Frames

----------

Standard frames are available in five sizes and four different pad

counts, for a total of ten standard frames as listed in the tables

below.  The matrix shows the official "name" assigned to each frame

offered.  Each frame size represents the total area available to the

user for project geometry; all scribe lanes, markings, and other

manufacturing and processing details are added by MOSIS outside this

frame.

Frame          ----------------Package---------------

(microns)        28          40          64          84

------------

7900 x 9200       --          --       64P79X92    84P79X92

6900 x 6800       --       40P69X68    64P69X68    84P69X68

4600 x 6800       --       40P46X68    64P46X68       --

2300 x 3400    28P23X34       --          --          --

There are special standard pad frames for MOSIS Tinychips.  You must

use one of these frames to have your Tinychip parts bonded.  (You may

use these standard frames on non-Tinychip runs, too).

Tiny Chip

Frame          ----------------Package---------------

(microns)        28          40          64          84

------------

2300 x 3400    28PC23X34      --          --          --

2220 x 2250       --       40PC22X22      --          --

The 28-and 40-pin packages are both 0.6" wide DIPs; the 64-pin package

is a 0.9" DIP; the 84-pin package is a pin grid array.

The pad layout of each standard frame has one-fourth of the pads

evenly spaced along each side.  The following table characterizes the

pad layouts in terms of three parameters; the units are microns.

Pad-to-Edge     the  distance from the center of each pad to the adjacent frame

boundary.    Pad-to-Edge  is  intended  to  be  sufficient   to

accommodate the pad itself and a ground bus around the outside;

the  minimum  pad-to-edge  value  is  sufficient to accommodate

MOSIS standard library pads.

Pad-to-Pad      the (center to center) distance between  pads  along  an  edge.

The  minimum value is 256 microns (for the next round of larger

packages, the spacing will necessarily decrease  to  about  200

microns or so).

Pad-to-Corner   the  distance  from the center of the first/last pad on an edge

to the corner of the frame.  Where there is sufficient room,  a

minimum corner spacing of 750 microns is maintained.

The first number, pad-to-edge, is the same for all the pads in a

frame, but the other two are set separately for each pair of sides.

The described orientation has N(orth) corresponding to the +Y

direction of the Cartesian coordinate system used in CIF, E(ast) to

the +X direction, W(est) to the -X direction, and S(outh) to the -Y

direction.

Frame                 -----N/S-----         -----E/W-----

(Name)      Edge       Pad   Corner          Pad   Corner

---------      ---       ---      ---          ---      ---

28P23X34      150       256      382          300      800

40P46X68      150       300      950          500     1150

64P46X68      150       256      380          300     1150

40P69X68      200       500     1200          500     1150

64P69X68      200       300     1200          300     1150

84P69X68      200       300      450          300      400

64P79X92      250       400      950          500      850

84P79X92      250       300      950          375      850

These numbers are reflected in the CIF and LIST files described below.

All the pads of a standard frame project are bonded to the package

pads in a fixed pattern.  The DIP frames number pads beginning at #1,

which is at, or immediately above, the center of the E(ast) or +X

side, and proceeding counterclockwise around the frame.  The pin grid

array frames locate pad #1 at the E(ast) end of the N(orth) side and

proceed counterclockwise, terminating at the N(orth) end of the E(ast)

side.  The CIF files distributed by MOSIS highlight pin #1.

Note that MOSIS will connect one of the package pins to both the silicon

substrate and its associated project pad.  For the 28, 40, and 64 pin DIPs,

the substrate connection is made on pin #1; for the 84 pin PGA, it is on pin

#10, which is near the middle of the N(orth) side.

Procedures for using standard frames

------------------------------------

To use a standard frame, the user must:

1. acquire from MOSIS (or derive from the table above) a description of

the frame(s) of interest;

2. integrate that description into a design (i.e., put the pads at  the

right places);

3. inform  MOSIS  of  the  standard  frame  that  is  associated with a

submitted project.

Standard frame descriptions are available from MOSIS in two forms: (1)

a CIF file (framename.CIF) that draws the bonding pads and the frame

outline and highlights pad #1, and (2) a text file (framename.LIST)

that lists the bonding pad centers, one per line.  These files may be

obtained via a library request to MOSIS.  For example, the files

for 28P23X34 are obtained with the following request:

REQUEST: LIBRARY

LIBRARY: STD_FRAMES

FILES:	28P23X34.CIF, 28P23X34.LIST

REQUEST: END

When submitting to MOSIS a project using a standard frame, the

parameter "STD-FRAME" is mandatory; it may be included in any of the

NEW-PROJECT, SUBMIT, FABRICATE, or UPDATE requests.  The argument to

STD-FRAME is the frame name, for example:

STD-FRAME: 28P23X34

A project in a standard frame cannot specify a "PADS" parameter and

need not specify a "SIZE" parameter.  In addition, the "MIN-LAMBDA"

and "MAX-LAMBDA" parameters, which are used to control the limits of

project scaling, are irrelevant since standard frame projects are

never scaled.

The MOSIS CIF check for a standard frame project will include a check

that the project is not larger than the size of the declared frame and

that ALL the pads of the frame are indeed present.  The minimum

acceptable pad geometry is a 90 x 90 micron glass cut box over a 100 x

100 micron metal box, centered at the designated pad location.  A pad

may be larger and/or off-center, provided that it completely covers

the minimum geometry.

Project geometry may touch the frame boundary; MOSIS will provide

appropriate separation between adjacent frames.  However, when a

project is smaller than the frame size, MOSIS will figure the best fit

match between the actual project pads and those expected for the

frame.

There is no requirement anywhere that a project be specified in terms

of an absolute CIF origin.  All sizes and positions are relative to an

arbitrary project origin.  Following is a sample CIF file:

( 28P23X34.CIF -- MOSIS standard frame with 28 pads

(7 per side) in a 2300 x 3400 micron project frame.

Side:   2300    3400

N/S     E/W

----    ----

Pad-to-Pad:     256     300

Pad-to-Corner:  382     800

Pad-to-Edge:        150

);

DS 1 100/1;   (one minimum bonding pad, center at 0,0);

L NM; B 100 100 0 0;

L NG; B  90  90 0 0;

DF;

DS 3 100/1;   (pad #1, marked for visibility);

C 1;

L NX; W 4 -35 10 5 10; W 4 -35 -10 5 -10;

W 4 -25 20 -25 -20; W 4 -5 20 -5 -20;

W 4 15 30 25 30 25 -30; W 4 15 -30 35 -30;

DF;

DS 2 100/1;

L NX; B 2300 3400 1150 1700; (The frame,

from 0,0 to 2300,3400);

(  east side --  );

C 3 T 2150 1700; (pin #1 -- the substrate connection);

C 1 T 2150 2000; (pin #2);

C 1 T 2150 2300; (pin #3);

C 1 T 2150 2600; (pin #4);

(  north side --  );

C 1 T 1918 3250; (pin #5);

C 1 T 1662 3250; (pin #6);

C 1 T 1406 3250; (pin #7);

C 1 T 1150 3250; (pin #8);

C 1 T  894 3250; (pin #9);

C 1 T  638 3250; (pin #10);

C 1 T  382 3250; (pin #11);

(  west side --  );

C 1 T  150 2600; (pin #12);

C 1 T  150 2300; (pin #13);

C 1 T  150 2000; (pin #14);

C 1 T  150 1700; (pin #15);

C 1 T  150 1400; (pin #16);

C 1 T  150 1100; (pin #17);

C 1 T  150  800; (pin #18);

(  south side --  );

C 1 T  382  150; (pin #19);

C 1 T  638  150; (pin #20);

C 1 T  894  150; (pin #21);

C 1 T 1150  150; (pin #22);

C 1 T 1406  150; (pin #23);

C 1 T 1662  150; (pin #24);

C 1 T 1918  150; (pin #25);

(  east side again --  );

C 1 T 2150  800; (pin #26);

C 1 T 2150 1100; (pin #27);

C 1 T 2150 1400; (pin #28);

DF;

C 2;

E

Following is a sample LIST file:

<<<< 28P23X34.LIST >>>>

2150, 1700  -- pin #1 -- the substrate connection

2150, 2000  -- pin #2

2150, 2300  -- pin #3

2150, 2600  -- pin #4

1918, 3250  -- pin #5

1662, 3250  -- pin #6

1406, 3250  -- pin #7

1150, 3250  -- pin #8

894, 3250  -- pin #9

638, 3250  -- pin #10

382, 3250  -- pin #11

150, 2600  -- pin #12

150, 2300  -- pin #13

150, 2000  -- pin #14

150, 1700  -- pin #15

150, 1400  -- pin #16

150, 1100  -- pin #17

150,  800  -- pin #18

382,  150  -- pin #19

638,  150  -- pin #20

894,  150  -- pin #21

1150,  150  -- pin #22

1406,  150  -- pin #23

1662,  150  -- pin #24

1918,  150  -- pin #25

2150,  800  -- pin #26

2150, 1100  -- pin #27

2150, 1400  -- pin #28