Date: 8 December 1990 9:32:07

From: The MOSIS System <XMOSIS@CHEETA.ISI.EDU>(send requests to MOSIS@MOSIS.EDU)

Subject: Topic:  Library: SCP20_PADS, File: SCP20_PADS.DOC

To: evans@wiener.tisl.ukans.edu

(			*** NOTICE ***

These pads have been validated by the MOSIS System of USC/ISI. Any

modification of these pads by the user is done solely at the user's

risk.  Modified pads are not to be referred to as "MOSIS Pads" unless

prior agreement is obtained from USC/ISI.

MOSIS really intends that you use the complete (stuffed) digital or

analog pad frames without modification.  The individual pads and pad

frame subcells are supplied only as a courtesy to those who want to

build and verify their own custom pad frames.  These pads were not

designed to be interchangeable within the pad frame, rather, they were

optimized for power and size, and require intricate editing and

verification in different arrangements.);

*************************************************************************

*									*

*		TinyChip Pads Set - SCP (P well) 2 Micron		*

*									*

*************************************************************************

------------

INTRODUCTION

------------

This 2 micron SCP (P-well) TinyChip pads set was designed with the

MOSIS scalable CMOS design rules (rev 6) at a lambda of 1 micron.

You should obtain the newest Magic technology (dated no earlier than

7/1/88) from MOSIS to use these pads. All pads are laid out on a lambda

grid and are designed to be used with the 40PC22X22 Standard Frame.

When submitting a project to MOSIS using this pad set, you must specify

the technology as 'TINY-SCP' or 'TINY-SCE' (if you wrote the CIF file with

both wells).

------------------------

CONTENTS OF THE PADS SET

------------------------

IN.CIF

IO.CIF

OUT.CIF

LL.CIF		lower left GND corner pad

UR.CIF		upper right VDD corner pad

GND.CIF

VDD.CIF

BLANK.CIF

ANALOG.CIF	analog (straight thru) pad

SCP20_PADS.CIF	contains all the cells in the library (excluding stuffed

pad frames)

40PC22X22_STUFFED.CIF	contains a complete io pad ring layout, 2220x2250 um

40PC22X22_STUFFED_ANALOG.CIF	contains a complete analog pad ring layout,

2220x2250 um

-------------------

DESCRIPTION OF PADS

-------------------

The single most important pad in this pad set is the IO pad. From this

tri-state I/O pad we derived the IN pad and the OUT pad. A circuit

diagram for this tri-state I/O pad is given at the end of this

document.  Connecting ENABLE to GND will turn this I/O pad into an

input pad.  Connecting ENABLE to Vdd will result an output pad.  If

you use a signal to drive the ENABLE, this signal should be an active

signal (ie. a output from a static gate instead of output of a

transmission gate or pass transistors).

ESD protection is done with (1) Field Oxide transistor of size W/L> 500/3

microns, (2) 150 ohms N_diffusion resistor/diode (3) Tri-state output

drivers as pair of diode clamps.

------------------

PROPERTIES OF PADS

------------------

--------------

PHYSICAL SIZE

--------------

These pads should fit into the standard frame 40PC22x22 with mirroring.

They can also be used for other MOSIS standard frames. V4io, V4in, V4out,

Vblank, V4vdd, and V4gnd has none-well maximum bonding box (MBB)

of 200x210 microns each. The well MBB is 210x218 microns. The stuffed

frame has a non-well MBB of 2220x2250 microns. There are 9 pads on

each edge plus 4 corners used as Vdd and GND pads. The top edge has a

GND pad in the middle and its 2 corners are Vdd pads, while the bottom

edge has a Vdd pad in the middle and its 2 corners are GND pads. This

gives 3 Vdd and 3 GND pads total.

--------------

SIGNALS

--------------

IN :   In (unbuffered)

OUT:   Out

IO :   In, Out, and Enable

-------------------

CIRCUIT DIAGRAM

-------------------

+-----+

+----| PAD |

|    +-----+

+-+       |

Vdd                         |         \

+---------+                     +-+       / 150 ohm

|         |                       |GND    \

|+--+         +--+|  ENABLE                   |

+--0||               ||0---------         Vdd     |

|   |+--+         +--+|                    |      |

|       |         |                    |+--+      |

|       +---------+-------------------0|| P       |

|       |         |                    |+--+      |

ENABLE  |   |+--+         +--+| ENABLEbar          |      |

---------|---||               ||0--+------          +------+

|   |+--+         +--+|   |                |      | IN_unbuffered

|       |         |       |            |+--+      |

|       +---------+-------|------------|| N

|       |         |       |            |+--+

OUT   |   |+--+         +--+|   |                |

------+---||               ||---+              GND

|+--+         +--+|

|         |

+---------+

GND

--------------

DELAY

--------------

OUTPUT Driver Load (MEASURED)

no load  10pF    22pF     47pF

------------+--------------------------------

prop. delay |  5.1ns  7.9ns   12.5ns   18.1ns

(0 to 1)   |

------------+--------------------------------