Final Report

Performance Tuning Study and Tools for Computer Systems

Connected to the DREN

- Michael D. Linhart, Joseph B. Evans, Roel Jonkman, Jason Keimig

- Information and Telecommunication Technology Center

The University of Kansas

##### Nichols Hall, 2291 Irving Hill Road

##### Lawrence, KS  66045

Email: evans@ittc.ukans.edu

http://www.ittc.ukans.edu/projects/DREN/

March 1999

# Abstract:

This report presents a summary of the network performance measurements from various DREN computing resources during the period between May 1998 and October 1998.  The three MSRC centers studied include ASC, CEWES, and NAVO.

Various network information-gathering tools were used to select various high-speed network interfaces to use for the experiments.  Among these tools are netstat, ifconfig, traceroute, and ping.

Performance testing was accomplished using several pieces of software designed for network performance testing.  Among these are ttcp, NetSpec, netperf, standard ftp, and ftp/inetd modified for large TCP window sizes.  UDP tests were performed to determine the theoretical maximum network and machine speed.  UDP tests were also used to show a long-term throughput and loss graph.  This test was conducted for a period of 3 days with measurements taken every 15 minutes.

One of the goals of these tests is to determine current site to site network performance statistics and suggest ways in which to optimize the performance.  Each architecture was researched for varying techniques to increase network throughput.  Very few network performance test applications would run on differing architectures without modification.

Values recorded from the testing were sometimes difficult to repeat.  Since the network and machines are constantly used, these tests represent varying conditions.  We did not have access to SNMP packet statistics at the edge routers, so we were unable to correlate the change in network performance due to a change in network conditions.

# 1.0:  Introduction

This report describes the performance measured on the specified DREN sites listed in section 2.0.  Several different software methods were used to measure such things as TCP performance, UDP performance, and ftp performance.  Not all software tools would function on all computer architectures.  Some software components needed to be modified to function correctly from different models or operating systems on the same architecture.

Performance results varied depending on variables such as the speed of the network interface cards, the load on the machine, and perceived network load.  Not all variables were able to be determined without more data for the machines and data from network routing statistics.

For most tests, data was generated on the fly so that the network file system delays would not affect the test results.  When ftp tests were used, data was sent from and to a temporary directory, which usually resides locally on the machine.  Since the purpose these tests was to determine the performance of the network, they were not gentle to the network.  On one such system, we were asked by the system administrator to please discontinue testing due to FDDI log errors and sluggishness of the machine.  Another machine more suitable was suggested.

# 2.0: Sites and Topology

The following Major Shared Resource Center (MSRC) machines were used for network performance testing.  Some machines listed here have been decommissioned, and others not listed have taken their place.

Aeronautical Systems Center  (ASC)

- sd2.asc.hpc.mil (SUN)

- hpc01.asc.hpc.mil (CRAY C90)

- hpc02.asc.hpc.mil (IBM Sperry)

- hpc03.sp.asc.hpc.mil (O2K)

- hpc07.asc.hpc.mil (SGI Power Challenge)

U.S. Army Corps of Engineers Waterways Experiment Station (CEWES)

- pk.wes.hpc.mil (CRAY C90)

- pca1.wes.hpc.mil (SGI)

- pca2.wes.hpc.mil (SGI Power Challenge)

- msf.wes.army.mil (CRAY J9)

- jim.wes.hpc.mil (CRAY T3E)

- osprey.sp.wes.hpc.mil (IBM Sperry)

- origin.wes.hpc.mil (SGI)

Naval Oceanographic Office (NAVO)

- odyssey.navo.hpc.mil (SGI)

- vlsc.navo.hpc.mil (CRAY C90)

- pca.navo.hpc.mil (SGI PCA)

- msas1.navo.hpc.mil (CRAY J90)

- neptune.navo.hpc.mil (CRAY J932)

- oceanus.navo.hpc.mil (SGI)

- seymour.navo.hpc.mil (CRAY T3E)

# 3.0: Network Performance Measurement

In this section, the methods and tools that have been used for obtaining the measurements are discussed.  Data was collected in the form of TCP and UDP bit throughput per second.  UDP measurements were done while varying the write buffer size.  TCP measurements were done while varying the write buffer size and TCP window size.  A variety of high-speed network interfaces were used, including HIPPI, FDDI, and ATM.  Tests were done on varying high-speed interfaces when possible.  Not all network interfaces were able to route to other sites for testing.  Experiments were typically performed 3 times and the average was taken for the report.  Large variations in performance were seen for the same tests.  Some patterns were dependent on time of day, and others were dependent on machine or perceived network load.

Figure 3.1: UDP test between CEWES and ASC.

- Figure 3.2: UDP extended tests from ASC to CEWES.

As seen in Figure 3.2 above, there is cyclical pattern per day for both the transmit and receive data.  This may be caused by some cyclic CPU and I/O intensive processes such as a nightly backup.  The data in Figure 3.1 and Figure 3.2 was measured with NetSpec.  Typically backups are done at night and may be responsible for the lowered throughput rates seen after midnight.

Figure 3.3: UDP test between NAVO and ASC.

As can be seen in Figure 3.3, optimal write buffer sizes are between 512K and 1024K.

TCP performance tests at CEWES between pca1 and pca2 yielded a throughput of 80.4 Mbps.  These tests were performed across the FDDI network interfaces and give an idea of the maximum throughput for these type of machines and FDDI network interfaces.

Figure 3.4: TCP test at NAVO

As can be seen in the results of Figure 3.4, the network test results were not always symmetrical.  These tests were performed within minutes of each other, but show a significant throughput difference.

FTP tests were performed on several machines.  The FTP testing was accomplished with the standard FTP supplied on each machine.  These tests may vary in TCP options such as the default TCP window size depending on the end architectures.  A duplicate test file of approximately 20 Megabytes was used to conduct the tests.

Figure 3.5 shows the large differences found when running FTP over the DREN network.  It is not surprising that the highest throughput values shown are results from FTP tests between machines at the same DREN site.

Figure 3.5: FTP throughput Tests

Figure 3.6: Average FTP throughput Tests

Figure 3.7: Best ftp throughput site to site

Several UDP throughput tests have been conducted by using NetSpec and/or ttcp.  Some of the collected data is displayed in Table 3.1.  Some variations in shown rates are a result of adjusting network parameters such as socket buffer sizes.

| UDP Throughput Tests |  |  | Sent | Received |
| --- | --- | --- | --- | --- |
| Source/Destination |  |  | Mbps | Mbps |
| NAVO to CEWES |  |  |  |  |
| odyssey to origin |  |  | 95.86 | 48.87 |
| odyssey to origin |  |  | 95.86 | 62.23 |
| odyssey to origin |  |  | 95.87 | 62.23 |
| ASC to CEWES |  |  |  |  |
| sd2 to pca2 |  |  | 44.62 | 43.85 |
| sd2 to pca2 |  |  | 45.93 | 44.02 |
| sd2 to pca2 |  |  | 46.05 | 45.00 |
| sd2 to pca2 |  |  | 48.22 | 2.99 |
| NAVO to ASC |  |  |  |  |
| pca to sd2 |  | 49.43 | 42.14 |

Table 3.1: UDP Throughput Tests

Several TCP throughput tests have been conducted by using NetSpec and/or ttcp.  Some of the data is displayed in Table 3.2 below.  Some variations in shown rates are a result of adjusting network parameters such as TCP window sizes.  A more complete table and graph will be provided in the final report.

| TCP Throughput Tests |  |  | Mbps |
| --- | --- | --- | --- |
| CEWES to ASC |  |  |  |
| pca1.wes to sd2.asc |  |  | 4.63 |
| CEWES to ASC |  |  |  |
| pca2.wes to sd2.asc |  |  | 5.77 |
| CEWES to ASC |  |  |  |
| pca2 to sd2 |  |  | 1.63 |
| pca2 to sd2 |  |  | 9.05 |
| pca2 to sd2 |  |  | 5.77 |
| ASC to CEWES |  |  |  |
| sd2 to pca2 |  |  | 7.43 |
| sd2 to pca2 |  |  | 6.63 |
| NAVO to ASC |  |  |  |
| odyssey to hpc03 |  |  | 24.16 |
| odyssey to hpc03 |  |  | 30.77 |
| odyssey to hpc03 |  |  | 38.11 |
| odyssey to hpc03 |  |  | 32.99 |
| odyssey to hpc03 |  |  | 21.48 |
| odyssey to hpc03 |  |  | 21.01 |
| odyssey to hpc03 |  |  | 45.42 |
| odyssey to hpc03 |  |  | 28.50 |
| odyssey to hpc03 |  |  | 26.18 |
| odyssey to hpc03 |  |  | 30.91 |
| odyssey to hpc03 |  |  | 18.16 |
| NAVO to NAVO |  |  |  |
| odyssey to pca |  |  | 62.45 |
| odyssey to pca |  |  | 71.35 |
| CEWES to CEWES |  |  |  |
| pca2 to pca1 |  |  | 71.27 |
| pca2 to pca1 |  |  | 72.52 |
| pca2 to pca1 |  |  | 75.59 |
| pca2 to pca1 |  |  | 22.86 |
| pca2 to pca1 |  |  | 25.17 |
| pca2 to pca1 |  |  | 24.54 |
| CEWES to CEWES |  |  |  |
| pca1 to pca2 |  |  | 84.41 |
| pca1 to pca2 |  |  | 26.43 |
| pca1 to pca2 |  |  | 27.18 |
| pca1 to pca2 |  |  | 19.36 |

Table 3.2: TCP Throughput Tests

# 4.0: Application Level Performance Issues in Large Bandwidth-Delay Networks

Most commonly used Internet applications (FTP, WWW) are not designed with large bandwidth-delay networks in mind.  This section will try to explain the peculiar problems associated with these types of networks, and the relatively small modifications that have to be implemented to alleviate the constraints.

A characteristic of large bandwidth-delay product networks that affects window-based protocols such as TCP is that the large amount of data that must be 'in flight' to provide the expected throughput.  Most default configurations do not provide windows that are sufficient for allowing large amounts of data to be 'in flight'.

The amount of data that can be in flight, and the resulting minimum window that is required to effectively use the bandwidth, is

Minimum window = Bandwidth * Delay

For a simple 10 Mbps Ethernet going across a typical campus (less than 10 ms), this would be

12,500 bytes = 106/8 bytes/second * 0.01 seconds

For an ATM OC3 (135 Mbps effective) spanning the US (approximately 100 ms), this would be

1,687,500 bytes = 135*106/8 bytes/second * 0.1 seconds

At least 1.6 MB of data would thus need to be 'in flight' in order to effectively use TCP applications over such a transcontinental link.  Unfortunately, the defaults that most operating systems and applications provide fail to furnish this.  TCP by design has a maximum window of 64 kB, due to the fact that the TCP uses 16 bit counters to indicate the acknowledgement bytes.  In certain circumstances the 16 bit counter is implemented as a signed number, and in these cases is even further limited to 32 kB.  RFC1323 (Jacobson, Braden, and Borman) describes the extensions that circumvent this limitation. The main feature that we need is what is called window scaling.  Instead of using byte counters for the acknowledgement window, the counters are scaled to acknowledge in multiples (the scaling is done in factors that are powers of 2).  Obviously, this RFC led to some modifications to TCP implementations within the past 2 to 3 years.  Many operating systems include the window scaling feature, either as a patch (Solaris 2.5.1 and lower, for example) or as part of the standard distribution.  On some machines, the TCP window scaling or long window option is something that must be configured or patched before compiling the kernel.  The large window scale option seems to be set active by default for the operating system versions on the DREN HPC machines, so no change is required.

One of the implications of the way that window scaling is typically implemented, however, is that the application kernel interface needs to specify a buffer size that is sufficient to hold the data that the window requires.  Most system utilities do not allow the user to adjust socket buffer sizes.  As networks get faster, the need will increase for applications that support socket buffer size adjustments.  BSD sockets are one of the most commonly used programming interfaces for Internet applications. BSD sockets use two buffers, the transmit and receive buffer respectively.  These two buffers implicitly determine the upper bound on the maximum sizes of the TCP transmit and receive windows.  System calls are used to query and set these socket buffer sizes.  The receive buffer system calls are

getsockopt (socketDescriptor, SOL_SOCKET, SO_RCVBUF, ptr2BufferSize, 4)

setsockopt (socketDescriptor, SOL_SOCKET, SO_RCVBUF, ptr2BufferSize, 4)

and the transmit buffer system calls are

getsockopt (socketDescriptor, SOL_SOCKET, SO_SNDBUF, ptr2BufferSize, 4)

setsockopt (socketDescriptor, SOL_SOCKET, SO_SNDBUF, ptr2BufferSize, 4)

where socketDescriptor identifies the socket and ptr2BufferSize is a pointer to a buffer size value.  In order to customize the socket buffer sizes within an application, whenever a socket is created, the buffer sizes need to be 'customized' using these system calls.  The values are determined by the expected network diameter (round trip time).

During operation, the window size that TCP uses is a dynamic value that is anywhere in the range from 1 MTU up to the bound that is determined by the socket buffer size.  Several strange side effects can be noticed when using TCP in extremely long networks is that the TCP slow start algorithm initially opens the connection with only 1 MTU worth of window, and exponentially ramps this up.  The resulting time it takes to achieve full bandwidth utilization is detrimental for short-lived connections.  Further enhancements to protocols have improved this, but this remains a somewhat problematic point.  An implication of this is that bandwidth estimation tests should be run for longer duration, anywhere from 30 seconds up to 2 minutes.

One of the side effects of using large socket buffers is that in certain cases one actually sees memory performance limitations rather than the effects of actual network performance.  The reason for this is that the kernel uses the socket buffers to store as much data as the application offers, and is not directly controlled by the dynamics of the actual TCP window.  This can be seen whenever application level measurements are not averaged, but rather shown over the duration of the connection.  If not carefully interpreted, the application measurements can therefore be somewhat deceiving.

The above application and operating system modifications are relatively painless and simple when the appropriate source code is available (or vendors have been kind enough to include these improvements).  We have collected the source code for the applications discussed in this report (where possible).  These are summarized with relevant pointers at

http://www.ittc.ukans.edu/projects/DREN/software/

A tunable version of an FTP client and server from NCSA were used to measure performance while adjusting TCP options such as the TCP window size.  Further modifications were done to allow compiling on the various operating systems at DREN HPC sites.  A similar FTP application was developed at NASA for high-speed network testing, but this has not been generally available to the community.  The objective of our effort is to provide a tool which may be freely distributed and which is portable to a variety of architectures such as those in use in the DoD HPC environment.

# 5.0: Network Testing Software

Many network testing software applications were tried and several were used to collect data.  Some worked without modification on multiple architectures, while others required minor modifications.  These testing software applications were used to determine what user level application settings could be changed to increase network performance.  The following list of software is typically network performance testing software.  Typical end-user applications will be discussed in the next section.

# 5.1: NetSpec

NetSpec was used on several of the tests when permitted.  Below is an example of a NetSpec control script for one of the tests.

cluster {

test 204.222.177.35 {

type = full (blocksize=32768, duration=30);

protocol = tcp;

own = 204.222.177.35:42105;

peer = 204.222.177.42:42105;

}

test 204.222.177.42 {

type = sink (blocksize=32768, duration=30);

protocol = tcp;

own = 204.222.177.42:42105;

peer = 204.222.177.35:42105;

}

}

# 5.2: ttcp

ttcp was used in various versions.  ttcp has been rewritten or modified many times for different architectures and slightly different purposes.  We were unable to find a ttcp that would compile and function correctly on the architectures in question.  Since ttcp was relatively small and easy to modify, we used this test program most often for the UDP and TCP tests.  A version of ttcp written in Java is now available from NLANR, with a copy at http://www.ittc.ukans.edu/projects/DREN/software/bench/javattcp.tar.gz.  This may alleviate some of the portability issues, although the implications of the Java implementation on performance have not been fully investigated.

# 5.5: Netperf

Netperf was used on some architectures, but we found the modified ttcp applications to be less prone to specific architecture compile and runtime problems.

# 5.6: Netvar

Netvar was a built in application that would show the current network variable settings.  The values shown below are from hpc01.asc.hpc.mil on 9/11/98.  As can be seen, the TCP buffer send space and receive space is set to 32 Kbytes.

hpc01:/hafs8/mlinhart/ netvar

Network configuration variables

tcp send space is 32768

tcp recv space is 32768

tcp time to live is 60

tcp keepalive delay is 14400

udp send space is 65536

udp recv space is 68096

udp time to live is 60

ipforwarding is on

ipsendredirects is on

subnetsarelocal is on

dynamic MTU discovery is off

adminstrator mtu override is on

maximum number of allocated sockets is 1000

maximum socket buffer space is 409600

operator message delay interval is 5

Fri Sep 11 14:53:23 EDT 1998

# 5.7: Miscellaneous

A file was viewed in the Irix systems to determine default TCP window sizes.  This file was located at /var/sysgen/master.c.  This file is used when the kernel is compiled to specify multiple kernel configuration values.  An example of the network settings section is shown below.  These values are from pca2.wes.hpc.mil on 9/18/98.

unsigned long tcp_sendspace = 60 * 1024;        /* must be < 512K */

unsigned long tcp_recvspace = 60 * 1024;        /* must be < 512K */

/* TCP large windows (RFC 1323) control. */

int tcp_winscale = 1;

int tcp_tsecho = 1;

int tcp_mtudisc = 1;

int tcp_mtutable[] = {

65535,

65280,  /* HIPPI */

32768,

17914,

9180,   /* ATM */

8166,   /* 802.4 */

4352,   /* FDDI */

2002,

1492,   /* Ethernet/802.3 */

1006,   /* Arpanet */

508,

0 /* ZERO MUST TERMINATE THIS LIST! */ };

/* UDP maximum datagram size */

unsigned long udp_sendspace = 60 * 1024; /* must be < (64K - 28) */

/* UDP socket buffers: reserve space for (sendspace * recvgram) bytes */

unsigned long udp_recvgrams = 2; /* clamped to 64K/sendspace */

As can be seen above, the tcp sendspace buffer size is set to 60 kBytes.  This can be increased as high as 512 kBytes.  Several tests have been run on specific pairs of machines to determine the optimal value for this setting between DREN sites.  60 kBytes is appropriate for DREN intra-site transfers, but for DREN site to site transfers, this number is not sufficient.

It was often difficult, as each DREN machine was tested against another, to determine what the best combinations of settings were.  In several cases, one setting was appropriate for one DREN site to site transfer, and yet another setting was more appropriate for a different DREN site to site transfer.  This made it difficult to calculate optimum network values for each machine or architecture.  What was interesting to note was how the optimal settings would differ even for the same architecture or model of machine.  Some of this could be attributed to the differences in NIC speed connections, but others could not.  Additional factors, such as the machine loads and the state of the network between sites may have been the cause of the observed behavior.

# 6.0: Application Software

The following list of software is typically network performance testing software, except for Ncftp and Wu-ftpd.  Ncftp can be considered a user owned application.  Wu-ftpd is typically thought of as running as root and is not run by an end user.  Modifications were made to Wu-ftpd to allow it to be run as a user owned application.

# 5.3: wu-ftpd

A modified version of wu-ftpd was used to allow for TCP large buffer settings.  It was also modified to allow for standalone mode on an unused high port that a user program was allowed to access.  This way the tests could be run without interference with the standard ftpd running on most of the machines.  This was more difficult than originally foreseen due to the /etc/passwd checking.

# 5.4: ncftp

ncftp was modified to allow for large buffer sizes and more automated logins so that testing could be performed from scripts.

# 7.0: Notes and Comments

It was sometimes difficult to perform the same test on the various architectures due to the differences in compilers available and socket support on each of the architectures.

Throughout the traceroute tests, several routers were seen to exhibit limitations on ICMP TIME_EXCEEDED responses.  This made it difficult in discovering routes.  Below is an example.

odyssey.navo.hpc.mil
204.222.177.42
IRIX64 odyssey 6.5SE 03250013 IP27

odyssey% /usr/etc/traceroute odyssey2.navo.hpc.mil
traceroute to odyssey2.navo.hpc.mil (204.222.177.150), 30 hops max, 40 byte packets
1 204.222.177.37 (204.222.177.37) 1 ms 1 ms 2 ms
2 * * *
3 * * *
4 * * *
5 * * *
6 * * *
7 * * *
8 * * * …

Certain machines did not allow certain types of network testing.  Below are two examples:

Cray UNICOS (msf)
msf.wes.army.mil.
134.164.13.210

msf% /etc/traceroute pk.wes.hpc.mil
traceroute: icmp socket: Permission denied

odyssey% /usr/etc/ping 129.48.245.50
PING hpc03-1.asc.hpc.mil (129.48.245.50): 56 data bytes
36 bytes from 198.97.67.36: Communication prohibited for icmp_seq=0
36 bytes from 198.97.67.36: Communication prohibited for icmp_seq=1
36 bytes from 198.97.67.36: Communication prohibited for icmp_seq=2

Kerberos increased the complexity for getting around from site to site.  At times, the kerberos login password would simply change and a call to the help desk would need to be done in order to reset the password.  A likely cause was due to some expiration date for passwords, but the login machines never alerted us to the need to change passwords after a set time.

The kerberos login machine Bastille located at NAVO does not initiate a kerberos login with forward permission.  This required the user to reinitialize the kerberos account with kinit -f each time we logged into Bastille.  On the other hand, Wesgate located at CEWES would initiate a kerberos login with forward permission, so the extra step was not necessary.

On multiple occasions, we lost connectivity from one DREN site to another.  Usually this was restored quickly.  Other interesting events were the loss of two to four days worth of saved user data, which occurred several times.  This was likely due to some file system problem.  We were concerned about the backup schedule and RAID configuration.

# 8.0: Conclusions

It was sometimes difficult to perform the same test on the various architectures due to the differing program library requirements and C compilers supplied on each machine.  Some architectures did not allow certain network privileges needed to perform such tests as ping, traceroute, etc.  Other difficulties include network routes that would not return information to software such as traceroute.

Wide variations of performance were seen from testing several machines that were performed only minutes after each other.  This may be due to a change in CPU and I/O loads, network congestion, or process scheduling to mention a few.

It is clear that to reach peak performance over a high-speed network connection, typical network applications need to be modified and careful attention given to network routes when machines have multiple network interfaces.

The custom ftp client and server with support for TCP large buffers provide significantly better performance than standard applications over large networks such as the DREN.

# 8.0: References

[1]	http://www.ncsa.uiuc.edu/People/vwelch/net_perf/tcp_windows.html  Von Welch, University of Maryland Baltimore, A User's Guide to TCP Windows.

[2]	http://www.psc.edu/networking/perf_tune.html Pittsburgh Supercomputing Center, Enabling High Performance Data Transfers on Hosts (Notes for Users and System Administrators).

[3]	http://www.ncne.org/tool_repository.html The National Center for Network Engineering, Tools Repository.

[4]	http://www.ncsa.uiuc.edu//People/vwelch/net_perf/sgi.html Von Welch, University of Maryland Baltimore, SGI Network Performance.