Delay-Power Product

Q: So, an ideal digital technology would have BOTH very small propagation delay tp, AND very small power dissipation PD, right ?

A: True ! But, there is a problem.  Designing a “faster” (e.g., lower tp) digital gate usually requires greater power. And designing a gate to minimize power consumption usually slows down the digital device.

Propagation delay and power dissipation generally form a design trade off - improve one and you degrade the other !

To quantify how effective, or efficient a digital design technology is in terms of delay or power, we use the product of the propagation delay and the power dissipation:

The delay-power product (DP) !

The delay-power product is therefore defined as:

*   Note we could define either a static or dynamic delay-power product, depending on PD.

*   Note also the unit of the delay-power product—Joules !!

The delay-power product is a figure of merit for digital technologies.