Example: A BJT Circuit in Cutoff and Saturation

Determine all currents for the BJT in the circuit below.

First lets ASSUME that both BJTs are in saturation.

Thus, ENFORCE:

VEB1 =VEB2= 0.7 V

VEC1=VEC2=0.2 V

VBC1=VBC2 = -0.5 V

Now, lets ANALYZE this circuit!

10.0 V

5.7 V

1 K

i1

50 K

 = 100

Q2

iB2

5.3 V

 = 100

Q1

1.0 K

iC2

10 K

iC1

Hey this is easy!  For saturation, if we know one voltage, we know them all!

We know:     VB1 = 5.3 V     and     VE2 = 5.7 V

Therefore, we can use the ENFORCED conditions to find all voltages.

VE1 = VEB1 + VB1 = 0.7 + 5.3 = 6.0 V

VC1 = VB1 - VBC1 = 5.3 – (-0.5) = 5.8 V

VB2 = VE2  - VEB2 = 5.7 – 0.7 = 5.0V

VC2 = VE2 – VEC2 = 5.7 – 0.2 = 5.5 V

Now for the currents! From Ohm’s Law:

And therefore from KCL:

Now we must CHECK our assumptions.  For Q1:

iE1 = 3.98 mA > 0        iC1 = 0.58 mA > 0      iB1 = 3.4 mA > 0  

and

iC1 = 0.58 mA <  iB1 = 100(3.4)  = 340 mA  

So far so good ! Now for Q2:

iE2 = 5.48 mA > 0      iC2 = 5.5 mA > 0     iB2 = -0.02 mA > 0 X

Our assumptions our incorrect.  We must change our assumptions and start again

(Doh) !

Lets still ASSUME that Q1 is in saturation, but change our Q2 assumption to cutoff.

Therefore ENFORCE  iB2 = iC2 = iE2 = 0

10.0 V

5.7 V

As before, the voltages for Q1 are:

VE1 = VEB1 + VB1

= 0.7 + 5.3 = 6.0 V

VC1 = VB1 - VBC1

= 5.3 – (-0.5) = 5.8 V

1 K

i1

50 K

 = 100

iB2 = 0

5.3 V

 = 100

Q2

Q1

1.0 K

iC2 = 0

10 K

iC1

Also as before, we find that i1 = 4mA and iC1 = 0.58 mA.

But, the emitter current is now iE1 = i1 + iB2 = 4 + 0 = 4 mA,

and thus the base current has changed to:

iB1 = iE1 – iC1 = 4.00 – 0.58 = 3.42 mA

Recall that we know (or think we do) the currents for Q2:

iB2 = iC2 = iE2 = 0

Now, lets determine the voltages of Q2:

5.3 V

10.0 V

5.7 V

50 K

1 K

1.0 K

10 K

Q2

Q1

iC1

iC2 = 0

i1

iB2 = 0

 = 100

 = 100

It is evident that:

VE2 = 5.7 V

VC2 = 0 + 1 iC2 = 0

VB2 = VE1 + 50 iB2

=  6.0 V

Therefore:

VEB2 = VE2-VB2

= 5.7-6.0 = -0.3 V

VBC2 = VB2-VC2

= 6.0 – 0 = 6.0 V

Cross your fingers! Its time to CHECK again. For Q1:

iE1 = 3.98 mA > 0        iC1 = 0.58 mA > 0      iB1 = 3.4 mA > 0  

and

iC1 = 0.58 mA <  iB1 = 100(3.4)  = 340 mA  

So far so good ! Now for Q2:

VEB2 = -0.3 V < 0            VBC2 = 6.0 V > 0 

Success ! Our assumptions are correct (woo-hoo!).