The Ideal

Operational Amplifier

We begin by considering the equivalent circuit of an ideal op-amp:

Note that output voltage is defined with respect to ground potential, while the input voltage is simply the potential difference between the plus (+) terminal and the minus (-) terminal.

Of course, we have three parameters in this circuit model: input resistance, output resistance, and open circuit voltage gain.

Q: So what are the ideal attributes (Rin, Rout, and Avo) of an operational amplifier?  In other words, what is the perfect op-amp?

A1:	The input resistance of a perfect op-amp is infinitely large (i.e., ).

A2:  The output resistance of a perfect op-amp is zero (i.e., ).

A3:  The open-circuit voltage gain of a perfect op-amp is very large, approaching infinity ()!

Thus, the equivalent circuit model of an ideal op-amp is:

Here we have changed the notation of the open-circuit voltage gain.  The value  is used, where:

In other words, the gain value  is unfathomably large!

Note then:

1.  Since the input resistance is infinite, the input current is zero—always!

2.  Since the output resistance is zero, the output voltage is equal to the open-circuit output voltage, even when the output load is not an open circuit!  I.E.,:

 regardless of !

Q:  What about the bandwidth of this “ideal” op-amp; is the model only valid for low-frequencies ?

A: Not for an ideal op-amp!  The bandwidth of an ideal op-amp is likewise infinite.

Q:  Wow! Unfathomably high voltage gain, infinite input resistance (impedance), zero output resistance (impedance), and:

regardless of the frequency spectrum .  This sounds like the perfect voltage amplifier!

A:  It is! That’s why we refer to it as the ideal op-amp.

Q: So why isn’t the input voltage with respect to ground potential?  Why is not the minus (-) input terminal connected to ground?

A:  Generally speaking, we find that two different voltages will be connected to the two different input terminals:

From KVL it is clear (right?) that the input voltage is:

And so the output voltage is:

Note that the input voltage is simply the difference between the two input signals  and .   We call this the differential input signal:

Thus, we can likewise express the output as:

Amplifiers of this type—where the input voltage is not defined with respect to ground—are referred to as differential amplifiers, as they can amplify the differential mode of two distinct signals (e.g.,  and ).

For example, say:

and

the ideal op-amp output voltage is therefore:

Note:

1.	The difference between  and  is amplified.

2.	The common signal (7 cos 10 t ) is eliminated by the subtraction .

Difference amplifiers ideally have perfect common-mode rejection. That is, the common signal between the two inputs has no effect on the output signal.

Of course, we can always connect a terminal to ground potential (i.e., ), thus making the input voltage a  value with respect to ground:

Q: I scoff at your so-called “ideal” op-amp.

Although 
 are obviously correct, I deem your assertion that Aop  should be unfathomably large (approaching 
) to be a silly notion.

After all, a gigantic gain Aop would mean that the output voltage 
 would likewise be unfathomably large—the destructive implications are obvious.

A: It is true that the output voltage will be very large—unless the differential voltage is unfathomably small!

For example, what if the differential voltage is approximately (i.e., almost)  zero:

?

In this case, the output voltage may not be very large at all!

Q:  Yes, but what is the likelihood that the two voltages  and  are nearly the same? This seems improbable.

A: Op-amps are generally not implemented by themselves!  Instead, they typically are but one component of many in a feedback amplifier.

In these applications, we will indeed find that —but we will also find that this is a desirable condition!

The condition   is know as a virtual short. If this is not true, the output voltage of an ideal op-amp will be unfathomably large.

As a result, the virtual short  is almost always the case in useful op-amp circuits