The Virtual Short

For feedback amplifiers constructed with op-amps, we have found (and will continue to find) that the two op-amp terminals will always be approximately equal ().

Of course, this must be true in order to avoid saturation, as the gain of an op-amp is ideally infinitely large.

Since  for feedback applications, it appears that the two op-amp terminals are shorted together!

+

-

vo

v-

v+

“Virtual Short”

The condition in op-amp feedback amplifiers where  is known as the “virtual short”.

Remember, although the two input terminal appear to be shorted together, they are most certainly not!

If a true short were present, then current could flow from one terminal to the other (i.e., ).  However, we know that the input resistance of an op-amp is ideally infinite, and thus we know that the input current  into an op-amp is zero.

v-

v+

+

-

vo

open

circuit

Therefore, it is not the op-amp that enforces the condition , it is the feedback that makes this so!

Applying the concept of a virtual short can greatly simplify the analysis of an op-amp feedback amplifier.  For example, consider again the inverting amplifier:

R2

+

-

i2

R1

v-

vin

ideal

v+

i1

This time, we begin the analysis by applying the virtual short condition:

Since the non-inverting terminal is grounded (v+ =0), the virtual short means that the non-inverting terminal is likewise at zero potential (v- =0)!

We refer to this condition as a virtual ground.

Analyzing the remainder of the circuit, we find:

R2

+

-

vin

ideal

R1

v-

v+

i1

i2

Combining, we find:

Rearranging, we again find the open-circuit, closed-loop voltage gain:

Note this is exactly the result we found before, yet in this case we never considered the op-amp equation:

The virtual short equation (
) replaced the op-amp equation in our analysis of this feedback amplifier—effectively, the two equations say the same thing (provided Aop is infinitely large, and we have negative feedback in the circuit).