# Special Problem 2.5-2

You design an amplifier with gain Avo =10, using an op-amp with a unity gain bandwidth of 10 MHz.

- What is the bandwidth of this amplifier?

A sinusoidal signal with frequency 10 kHz and magnitude of 1.0 is input to this amplifier.

2)	What is the magnitude of the output signal?

A sinusoidal signal with a new frequency of 10 MHz and magnitude of 1.0 is input to this amplifier.

3)	 What is the magnitude of the output signal?

You now redesign the amplifier such that the gain is now Avo = 20.

4)      What is the bandwidth of this amplifier?