Special Problem 2.5-3

The amplifier below has a 3 dB bandwidth of 100 kHz.

R2 =9K

R1 = 1K

+

-

vout

vin

i1

i2

1)	What is the gain-bandwidth product of the op-amp used in this amplifier?

2)	At what frequency is the magnitude of the amplifier gain equal to one ?

3)	What would the 3 dB bandwidth of this amplifier be if we changed R2 to 99 K?

4)	Say we replace the op-amp with one having a gain-bandwidth product of 100 kHz.  What is the 3 dB bandwidth of this amplifier if R2 = 9 K?