Output Voltage Saturation

Recall that the ideal transfer function implies that the output voltage of an amplifier can be very large, provided that the gain Avo and the input voltage vin  are large.

vout

vin

Avo > 0

Avo < 0

The output voltage is limited

However, we found that in a “real” amplifier, there are limits on how large the output voltage can become.

The transfer function of an amplifier is more accurately expressed as:

A non-linear behavior!

This expression is shown graphically as:

vout

vin

Avo

L+

L-

This expression  (and graph) shows that electronic amplifiers have a maximum and minimum output voltage (L+ and L-).

If the input voltage is either too large or too small (too negative), then the amplifier output voltage will be equal to either L+ or L- .

If vout = L+ or vout =L- ,  we say the amplifier is in saturation (or compression).

Make sure the input isn’t too large!

Amplifier saturation occurs when the input voltage is greater than:

or when the input voltage is less than:

Often, we find that these voltage limits are symmetric, i.e.:

For example, the output limits of an amplifier might be L+ = 15 V and L- = -15 V.

However, we find that these limits are also often  asymmetric (e.g., L+ = +15 V and L- = +5 V).

Saturation: Who really cares?

Q:  Why do we care if an amplifier saturates? Does it cause any problems, or otherwise result in  performance degradation??

A:  Absolutely!  If an amplifier saturates—even momentarily—the unavoidable result will be a distorted output signal.

A distortion free example

For example, consider a case where the input to an amplifier is a triangle wave:

vin (t)

t

vout (t)

t

Since  for all time t, the output signal will be within the limits L+ and L- for all time t, and thus the amplifier output will be vout (t) = Avo vin (t):

The input is too darn big!

Consider now the case where the input signal is much larger, such that  for some time t (e.g., the input triangle wave exceeds the voltage limits  and  some of the time):

vin (t)

t

This is precisely the situation about which I earlier expressed caution.

We now must experience the palpable agony of signal distortion!

Palpable agony

vout (t)

t

Note that this output signal is not a triangle wave!

For time t where , the value  is greater than L+ and less than L-, respectively.

Thus, the output voltage is limited to  for these times.

As a result, we find that output  does not equal  —the output signal is distorted!

Amplifiers with op-amps

For amplifiers constructed with op-amps,  the voltage limits L+  and L-  are determined by the DC Sources V + and V - :

and

+

-

vin

vout

ideal

R2

R1