Special Problem 2.8-1

The input to the circuit below is:

vi (t) = 0.5 t2 - 0.5

where time t is in microseconds.

The slew rate of the op-amp is 10 V/sec.

The DC power supplies are V+ = 15 V and V- = -15 V.

1) If R2 = 1 K,  at what time t will the output signal vo(t) begin to distort?

2) If R2 = 9 K,  at what time t will the output signal vo(t) begin to distort?

1K

1K

R2

vo

vi

non-ideal

op-amp

V-

V+