# Special Problem 2.8-2

Consider the following amplifier:

_

+

vi

9K

1K

15V

-15V

vo

The Slew Rate of the Op-Amp is 10V/s.

The D.C. power supply voltages are +/- 15 Volts.

The input signal vi is a triangle wave with period T and magnitude A:

vi

t

t=0

t=T

A

-A

- If  T = 4 sec, what is the largest value of  A that will produce a distortion-free output vo?

- If  T = 8 sec, what is the largest value of  A that will produce a distortion-free output vo?