Real Op-Amp Input and Output Resistances

The input resistances of real op-amps are very large, but of course not infinite!

Typical values of input resistances range from several hundred Kilo Ohms to tens of Mega Ohms.

As a result, there is a small amount of current flowing into input terminals of a real op-amp.

Q:  Well of course! We just studied this topic. We already know that there is a bias current IB flowing into (or out of) real op-amp terminals!

A:  This is true! However, there is an additional amount of current flowing into the input terminals.  This current is not a constant bias current, but instead is directly proportional to the input terminal voltage.

Because the input resistance is finite, the total current into real op-amp terminals are:

As such, our input terminal circuit model is:

-

+

Op-Amp Input Terminal Model

Ri

Ri

We find that the input current  or  will be insignificant (i.e., we can ignore its effect), provided that all other resistors used in an op-amp circuit are significantly less than the op-amp input resistance Ri.

Q:  But this would imply that we should never use resistor values greater than 100K in our op-amp circuits!

A:  That’s exactly right!  If the resistor values that you use in your op-circuit design are of the order of Ri, you may find that your circuit behaves quite differently from what you expected!

Now let’s examine the real values of op-amp output resistance. Instead of the ideal value of zero, we find that the output resistances of real op-amps are non-zero (i.e., )!

Recall that the output resistance of both the inverting and non-inverting configurations is approximately equal to the op-amp output resistance (i.e., ).  Thus, we find that the output resistance of real inverting and non-inverting amplifiers are likewise non-zero!

Q:  NO! The amplifier output resistance is not zero?!? This means that the amplifier output will not be equal to the open-circuit voltage if a finite load is attached!

A:  This is absolutely correct!  Remember, the output voltage of an amplifier is equal to the input voltage times the open-circuit voltage gain only when the amplifier output is connected to an open circuit.

But, recall that the output voltage will be approximately equal to the open-circuit voltage if the output resistance is much smaller than the load resistance.  I.E.,: