The Input Bias Current

-

+

Real op-amps typically exhibit a phenomenon known as input bias current.

We find that there is a small amount of current flowing into each of the op-amp inputs (i.e., , and )!

These currents are constant currents—in other words, they are independent of the input terminal voltage

Input Bias Current Op-Amp Model

Real op-amps act like there are small current sources at the inputs!!!!!!!

The input offset current

The values of bias currents IB1 and IB2 are approximately—but not exactly—equal.

As a result, we typically express these currents in terms of their common-mode (i.e., average) and differential modes.

The common mode is called the Input Bias Current:

The differential mode is called the Input Offset Current:

They seem so small, yet…

Thus, the two bias currents can be expressed as:

Typical values of these parameters are, for example, IB = 100nA and Ios =10nA.

Q: These bias current values are so tiny, we do we even care about them????

A:  Because they can cause offset voltages in op-amp circuits!