The Input Bias Current

Real op-amps typically exhibit a phenomenon known as input bias current.

We find that there is a small amount of current flowing into each of the op-amp inputs (i.e., , and )!  These currents are constant currents--in other words, they are independent of the input terminal voltage.

-

+

Real op-amps act like there are small current sources at the inputs!!!!!!!

Input Bias Current Op-Amp Model

Typically, the values of bias currents IB1 and IB2 are approximately, but not exactly, equal.

We therefore use these symbols and definitions to specify input currents:

Typical values of these parameters are, for example, IB = 100nA and Ios =10nA.

Q:  Hey! These bias current values are so tiny, we do we even care about them????

A:  Because they can cause offset voltages in op-amp circuits!