The Input Offset Voltage

-

+

For real op-amps, we typically find that if both inputs are grounded, the output will be—saturated!

Q:  What!? Why isn’t

??

A:  The reason the output is saturated is that real op-amps exhibit a phenomenon called the input offset voltage Vos.

The input offset voltage model

This value can be either positive or negative, typically with a magnitude of 5 mV or less ().

A real op-amp therefore behaves as if it has a small, internal voltage source at the non-inverting input:

-   +

ideal

+

-

-

+

Input Offset Voltage Equivalent Model

Applying the concept of a virtual short to the ideal op-amp, we find from KVL

Thus, !

The new virtual “short”

Recall, however, that the input offset voltage is typically very small (i.e., ), so that .

So, for an op-amp with an input offset voltage, the virtual “short” equation turns out to be:

Small, but large enough to saturate!

Therefore, if , we find that the output voltage of this op-amp is ideally equal to:

-   +

ideal

+

-

-

+

Of course, since the differential voltage Aop is very large, the product Aop Vos is likewise large, such that the output of real op-amps will saturate.

This changes our previous results

Q:  Does this mean that Vos will cause the output of op-amp circuits and amplifiers to saturate?

vout

vin

A

Voff

A: Fortunately no!

However, the input offset voltage will affect the output of circuits and amplifiers made with op-amps.