Special  Problem 4.6-2

For the circuit below, VG is the DC bias at the gate, vi is the small-signal input and vo the small-signal output. The  transistor is known to be in the saturation region.

The capacitor in the circuit is extremely large.

1)  Draw the resulting small-signal circuit.

2)  In terms of  transconductance gm, find the small-signal gain Avo = vo/vi  .

NOTE: Do not attempt any DC analysis; provide your solutions directly in terms of gm. Ignore the output resistance (i.e., ro = ¥).

RS

VDD

vi(t)

RD

vo(t)

VG