D.C Biasing using a Single Power Supply

The general form of a single-supply BJT amplifier biasing circuit is:

+

-

+

-

Generally, we have three goals in designing a biasing network:

1)  Maximize Gain

Typically, we seek to set the operating point of the BJT amplifier such that the resulting small signal voltage gain is maximized.

However, we sometimes seek to set the bias point such that the output resistance is minimized, or the input resistance is maximized.

2)  Maximize Voltage Swing

We seek to set the operating point of the BJT amplifier such that the maximum small signal output can a large as possible.   If we make too small, then the BJT will easily saturate, whereas if is too large, the BJT will easily cutoff.

3)  Minimize Sensitivity to changes in

Q: You’re kidding me right? We’re supposed to achieve all these goals with only four resistors?

Manufacturing and temperature variances will result in significant changes in the value . We seek to design the bias network such that the amplifier parameters will be insensitive to these changes.

A:  Actually, the three design goals listed above are often in conflict.  We typically have to settle for a compromise DC bias design.

Let’s take a closer look at each of the three design goals:

1)  Maximize Gain

Typically, the small-signal voltage gain of a BJT amplifier will be proportional to  transconductance :

Thus, to maximize the amplifier voltage gain, we must maximize the BJT transconductance.

Q:  What does this have to do with D.C. biasing?

A:   Recall that the transconductance depends on the DC collector current :

Therefore the amplifier voltage gain is typically proportional to the DC collector current:

We of course can’t decrease the thermal voltage , but we can design the bias circuit such that  is maximized.

To maximize , maximize

2)  Maximize Voltage Swing

Recall that if the DC collector voltage is biased too close to , then even a small small-signal collector voltage  can result in a total collector voltage that is too large, i.e.:

In other words, the BJT enters cutoff, and the result is a distorted signal!

To avoid this (to allow  to be as large as possible without BJT entering cutoff), we need to bias our BJT such that the DC collector voltage  is as small as possible.

Note that the collector voltage is:

Therefore  is minimized by designing the bias circuit such that the DC collector current  is as large as possible.

Hey hey! It looks like amplifier bias design is going to be easy.  We can both maximize transconductance 
and minimize the DC collector voltage 
 by maximizing the DC collector current 
!

Just a second!  We must also consider the signal distortion that occurs when the BJT enters saturation.  This of course is avoided if the total voltage collector to emitter remains greater than 0.7 V, i.e.:

Thus, to avoid BJT saturation—and the resulting signal distortion—we need to bias our BJT such that the DC voltage  is as large as possible.

To minimize signal distortion, maximize

3)  Minimize Sensitivity to changes in

We find that BJTs are very sensitive to temperature—specifically, the value of  is a function of temperature.

Likewise, the value of  is not particularly constant with regard to the manufacturing process.  We find that 100 otherwise “identical” BJTs will result have 100 different values of !

Both of these facts lead to the requirement that our bias design be insensitive to the value of .  Specifically, we want to design the bias network such that the DC bias currents (e.g., ) do not change values when  does.

Mathematically, we can express this requirement as minimizing the value:

Let’s determine this value for our standard bias network:

+

-

Q: Yuck! This looks like a disturbingly difficult circuit to analyze.

A:  One way to simplify the analysis it to use a Thevenin’s equivalent circuit.

Specifically, replace this portion of the bias circuit with its Thevenin’s equivalent:

We find that this equivalent circuit is:

+

_

The bias network can therefore be equivalently represented as:

+

-

+

_

If we ASSUME that the BJT is in active mode, then we ENFORCE the proper equalities and ANALYZE this circuit to find collector current IC:

We find therefore that:

Note then that:

In other words, if we wish to make the DC collector current insensitive to changes in , we need to make:

We of course could accomplish this by making the base resistance  small, but we will find out later that there are problems with doing this.

Instead, we can minimize the circuit sensitivity to changes in  by maximizing the emitter resistor .

To minimize , maximize

So, let’s recap what we have learned about designing our bias network:

1.  Make  as large as possible.

2.  Make  as large as possible.

3.  Make  as large as possible.

Seems easy enough! Let’s get started biasing BJT amplifiers!

Not so fast!  We have a serious problem.  To see what this problem is, write the KVL equation for the Collector-Emitter Leg of the Bias Network:

+

-

or

Maximize Avo by maximizing this term.

Minimize distortion by maximizing this term.

Minimize  sensitivity  by maximizing this term.

But the total of the three terms must equal this!

Q: Yikes! It’s like owing 3 really big guys $15 each, but having only $15 in your pocket.

What do we do?

A:  Split the total voltage 3 ways (give each guy $5).

I.E., set:

+

In other words, for an npn BJT, set:

+

-

Likewise, for a pnp BJT, set:

+

-

Q:  We have determined that the product  should be equal to .  We can of course accomplish this with a larger resistor RC and a smaller current IC, or a larger current IC and a smaller resistor RC.  What should the value of IC be?

A:  Generally speaking, the value of the DC collector current IC affects:

1)  Voltage Gain ().

2)  Input Resistance ().

3)  BJT Output Resistance ().

4)  Power Consumption ().

5)  Amplifier Bandwidth ().

The “best” value of collector current  is a trade between these parameters.

Q:  OK, we now have enough information to set , and thus resistors .  But we still have two bias resistors left--.  How do we determine there values?

A:  Well, we have found that reducing  decreases the circuit sensitivity to   This is good!

But, we will find that reducing  will often decrease the amplifier input resistance   This is bad!

Also, we find that reducing  will increase the power dissipation  This is also bad!

A general “rule of thumb” is to select the values of R1 and R2 so that IC is:

Remember, the resistors R1 and R2 also determine the base voltage VB, which should approximately be: