Amplifier Output Power

Say we have an amplifier with gain G = 30 dB (i.e., G = 1000).

If the input power to this amplifier is 0 dBw (i.e., Pin = 1W), then the output power is:

Or, in dB:

WOW!  We created 999 Watts !

The energy crisis is solved !

Of course, the amplifier cannot create energy.

Q:  Then, where does the power come from ???

A:  The D.C. power Supply ! (Every amplifier has one).

+V

Pin

Pout

I

D. C. Power = V I > Pout

The output power Pout cannot exceed the power delivered by the D.C. supply.

Q: What happens to the D.C. power not converted to signal power Pout ??

A:

So, if we were to plot Pout vs. Pin for a microwave amplifier, we would get something like this:

Pin

Pout

G

Pout = G Pin

D. C. Power

We notice that the output power compresses, or saturates.

Note there is one point on this curve where the amplifier output power Pout is 1 dB less than its ideal value of G Pin. In other words, there is one (and only one!) value of Pin and Pout that will satisfy the equation:

At this point, the amplifier is said to be compressed 1 dB. In other words, a 10 dB amplifier would appear to be a 9 dB amplifier!

The output power when the amplifier has compressed 1dB is called the 1 dB compression point  P1dB of the amplifier.

The 1 dB compression point is considered to be the maximum power output of the amplifier.

The input power at the 1 dB compression point is said to be the maximum input power () of the amplifier.  We of course can put more than  into the amplifier—but we won’t get any more power out!

Pin

Pout

G

Pout = G Pin

1 dB

P1dB

Note the equation  alone is not sufficient to determine the 1 dB compression point, as we have two uknowns (Pin and Pout).  We need another equation!

This second “equation” is the actual curve or table of data relating Pin to Pout for a specific amplifier.

Amplifier Efficiency

We can define amplifier efficiency e as the ratio of the maximum output power ( P1dB) to the D.C. power:

(don’t use decibels here!)

For example, if e=0.4, then up to 40% of the D.C. power can be converted to output power, while the remaining 60% is converted to heat.

We require high power amps to be very efficient!