Non-Linear Behavior

of Amplifiers

Note that the ideal amplifier transfer function:

is an equation of a line (with slope = Avo and y -intercept = 0).

vout

vin

Avo > 0

Avo < 0

This ideal transfer function implies that the output voltage can be very large, provided that the gain Avo and the input voltage vin  are large.

However, we find in a “real” amplifier that there are limits on how large the output voltage can become.  The transfer function of an amplifier is more accurately expressed as:

This expression is shown graphically as:

vout

L+

vin

Avo

L-

- This expression  (and graph) shows that electronic amplifiers have a maximum and minimum output voltage (L+ and L-).

- If the input voltage is either too large or too small (too negative), then the amplifier output voltage will be equal to either L+ or L- .

- If vout = L+ or vout =L- ,  we say the amplifier is in saturation (or compression).

Amplifier saturation occurs when the input voltage is greater than:

or when the input voltage is less than:

Often, we find that these voltage limits are symmetric, i.e.:

For example, the output limits of an amplifier might be L+ = 15 V and L- = -15 V.

However, we find that these limits are also often  asymmetric (e.g., L+ = +15 V and L- = +5 V).

Q:  Why do we care if an amplifier saturates? Does it cause any problems, or otherwise result in  performance degradation??

A:  Absolutely!  If an amplifier saturates—even momentarily—the unavoidable result will be a distorted output signal.

For example, consider a case where the input to an amplifier is a triangle wave:

vin (t)

t

Since  for all time t, the output signal will be within the limits L+ and L- for all time t, and thus the amplifier output will be vout (t) = Avo vin (t):

vout (t)

t

Consider now the case where the input signal is much larger, such that  for some time t (e.g., the input triangle wave exceeds the voltage limits  and  some of the time):

vin (t)

t

This is precisely the situation about which I earlier expressed caution.  We now must experience the palpable agony of signal distortion!

vout (t)

t

Note that this output signal is not a triangle wave!

For time t where , the value  is greater than L+ and less than L-, respectively.  Thus, the output voltage is limited to  for these times.

As a result, we find that output  does not equal  —the output signal is distorted!

In reality, the saturation voltages  are not so precisely defined.  The transition from the linear amplifier region to the saturation region is gradual, and cannot be unambiguously defined at a precise point.

vout

vin

Avo

L+

L-

vout(t)

t

Now for another non-linear problem!

We will find that many amplifiers exhibit a DC offset (i.e., a DC bias) at their output.

vout

Voff

vin

A

The output of these amplifiers can be expressed as:

where A  and Voff  are constants.  It is evident that if the input is zero, the output voltage will not be (zero, that is)!

Q: Yikes! How do we determine the gain of such an amplifier?

If:

then what is:

The ratio of the output voltage to input voltage is not a constant!

A: The gain of any amplifier can be defined more precisely using the derivative operator:

Thus, for an amplifier with an output DC offset, we find the voltage gain to be:

In other words, the gain of an amplifier is determined by the slope of the transfer function!

For an amplifier with no DC offset (i.e., ), it is easy to see that the gain is likewise determined from this definition:

Hey, hey! This definition makes sense if you think about it—gain is the change of the output voltage with respect to a change at the input.

For example, of small change 
 at the input will result in a change of 
 at the output.  If Avo is large, this change at the output will be large!

OK, here’s another problem.

The derivative of the transfer curve for real amplifiers will not be a constant.  We find that the gain of a amplifier will often be dependent on the input voltage!

The main reason for this is amplifier saturation.  Consider again the transfer function of an amplifier that saturates:

We find the gain of this amplifier by taking the derivative with respect to vin :

Graphically, this result is:

vin

vo

L+

Voff

L-

Thus, the gain of this amplifier when in saturation is zero. A change in the input voltage will result in no change on the output—the output voltage will simply be .

Again, the transition into saturation is gradual for real amplifiers.

In fact, we will find that many of the amplifiers studied in this class have a transfer function that looks something like this:

vin

vout

We will find that the voltage gain of many amplifiers is dependent on the input voltage.  Thus, a DC bias at the input of the amplifier is often required to maximize the amplifier gain.