Signal Expansions

Q: How is performing a linear operation easier than performing a non-linear one??

A:  The “secret” lies is the result:

Note here that the linear operation performed on a relatively complex element  can be determined immediately from the result of operating on the “simple” elements  and .

To see how this might work, let’s consider some arbitrary function of time , a function that exists over some finite amount of time T  (i.e., ).

Say we wish to perform some linear operation on this function:

Complex signals as collections

of simple elements

Depending on the difficulty of the operation , and/or the complexity of the function , directly performing this operation could be very painful (i.e., approaching impossible).

Instead, we find that we can often expand a very complex and stressful function in the following way:

where the values  are constants (i.e., coefficients), and the functions  are known as basis functions.

Ms. Nomial’s first name is Poly

For example, we could choose the basis functions:

Resulting in a polynomial of variable t :

This signal expansion is of course know as the Taylor Series expansion.

Choose your basis –

but choose wisely

However, there are many other useful expansions (i.e., many other useful basis ).

- The key thing is that the basis functions  are independent of the function .  That is to say, the basis functions are selected by the engineer doing the analysis (i.e., you).

- The set of selected basis functions form what’s known as a basis. With this basis we can analyze the function .

- The result of this analysis provides the coefficients  of the signal expansion.  Thus, the coefficients are directly dependent on the form of function  (as well as the basis used for the analysis).  As a result, the set of coefficients  completely describe the function !

It’s simpler to operate on each element

Q:  I don’t see why this “expansion” of function of  is helpful, it just looks like a lot more work to me.

A:  Consider what happens when we wish to perform a linear operation on this function:

Look what happened!

Instead of performing the linear operation on the arbitrary and difficult function , we can apply the operation to each of the individual basis functions .

Choose a basis that makes this “easy”

Q:  And that’s supposed to be easier??

A:   It depends on the linear operation and on the basis functions .

Hopefully, the operation  is simple and straightforward.

Ideally, the solution to  is already known!

Q:  Oh yeah, like I’m going to get so lucky.  I’m sure in all my circuit analysis problems evaluating  will be long, frustrating, and painful.

A: Remember, you get to choose the basis over which the function  is analyzed.

A smart engineer will choose a basis for which the operations  are simple and straightforward!

This basis is quite popular

Q:  But I’m still confused.  How do I choose what basis  to use, and how do I analyze the function  to determine the coefficients ??

A:  Perhaps an example would help.  Among the most popular basis is this one:

and:

So therefore:

The astute among you will recognize this signal expansion as the Fourier Series!

It has a very important property!

Q:  Yes, just why is Fourier analysis so prevalent?

A:  The answer reveals itself when we apply a linear operator to the signal expansion:

Note then that we must simply evaluate:

for all n.

We will find that performing almost any linear operation  on basis functions of this type to be exceeding simple (more on this later)!