412 Laboratory #5:  BJT Small-Signal Gain and Circuit Models

Objective:	To measure the values necessary for a small-signal equivalent circuit model of a BJT transistor.  To measure small-signal gain and use the equivalent circuit model to calculate the expected small-signal gain for comparison.  To understand the importance of a small-signal equivalent circuit model.

Components List:

(1) 1.5-k resistor, (1) 3.6-k resistor, (2) 2.2-k resistor, 
(1) 510- resistor, (2) 1-k resistors, (3) 2.2-F capacitors, (3) 0.1-F capacitors (power decoupling), (1) 3904 NPN transistor.

Procedure:

1.  Construct the following circuit on a breadboard (power decoupling not shown, but should be applied).

+15V

+15V

1 k

3.6 k

VC + vo(t)

1.5 kW

2.2 F

1.5 k

iB

vi (t)

- +

1 k

2.2 k

+

-

2.2 F

2.  Leaving vi (t) disconnected, measure the DC values of the collector voltage VC, and base current IB .

Q1:  Using the values measured in step 2, calculate the DC collector current IC and DC emitter current IE.

3.  Set vi (t) to be a 100-kHz, 100-mVpp, 0V offset sine-wave using the function generator.  Carefully measure and record the small-signal output, vo(t).

Q2:  Using these measurements, determine the apparent open-circuit small-signal gain (Avo=vo(t)/vi(t)) of this amplifier.

Q3:  Calculate the magnitude of the impedance of the capacitors in this circuit (at the small-signal frequency f=100 kHz). Compare this value to the resistors.  Can we replace these capacitors with a simpler circuit device?  If so, what would that device be?

Q4:  Use the values of IB and IC calculated in  Q1 to find gm and r.  Now use the BJT small-signal equivalent circuit to write the small-signal circuit for this amplifier.  Analyze this small-signal circuit and find the open-circuit small-signal gain, Avo.  Compare this with the measured value determined in step 3.

Q5:  Now analyze the small-signal circuit and calculate the small-signal output resistance of this amplifier.  What do you think about the value you calculated?  Is it a good value for a voltage amplifier?

- Now attach a 510- load to the output of the amplifier as shown below.  Carefully measure and record the output vo(t).

+

vo(t)

-

+15V

1 k

2.2 F

1 k

+15V

3.6 k

2.2 k

2.2 F

- +

+      -

vi (t)

1.5 k

2.2 F

510

+

_

Q6:  Now what is the apparent (measured) small-signal gain (it’s not the open-circuit gain!) Av=vo(t)/vi(t) ?  Analyze the circuit with the small-signal model and RL= 510  attached and calculate Av.  How do the measured and calculated values of Av compare?

Q7:  Compare the voltage gain you measured/calculated in for the first circuit (open-circuit output) to the voltage gain you measured/calculated for the second (with 510  load).  Explain in detail why the gains of the two circuits are different  (Hint:  see Q5).