LAB 3: MOSFET

| Title: | MOSFET |
| --- | --- |
| Objective: | To demonstrate the concept of large- and small-signal models, to determine the DC biasing, and to determine the small-signal parameters at low frequency, such as the voltage gain, input resistance, and output resistance for different circuit configurations. |
| Equipment and components: | 1 MOSFET (IRF 820); 2 resistors (k); oscilloscope; function generators; DC power supply. |
| Note: | Analyze the circuit first, which includes hand calculations or computer simulation. Compare your experiment measurement with your analysis predictions. |

Procedure

1. Set up the circuit shown below on a solderless board.

- Increase VG from 0 V to 5 V in step of 0.1 V, measure current ID and output voltage VO. Plot VGS vs. ID. Determine K and Vt. - [K can be obtained using the PolyFit function in Matlab. Plot the polynomial using the constants you obtained and compare it with the plot from your data. K will be the second order constant when the two plots fit closely. Use the K found to calculate Vt. Verify your Vt from the transistor data book.] - Adjust VG so that ID = 1 mA. Record VGS, VDS, and ID. Calculate gm. - Add vg = 0.4 sin ft with f = 1kHz in series with VG. Measure the small-signal output voltage on the oscilloscope and determine the voltage gain.

2. Set up the circuit shown below on a solderless board.

- With vg = 0 V, adjust VG so that ID = 1 mA. Record VGS, VDS, and ID. - Let vg = 0.4 sin ft with f = 1kHz. Calculate the ac input resistance rin. - Calculate the ac output resistance r0 looking into the output voltage vo. - Measure the small-signal output voltage on the oscilloscope and calculate the voltage gain.