8.  Build the following circuit on your breadboard.  You can remove the previous circuits.

10k

+

-

vI

High Output Resistance Source

LED

+

-

Low Resistance Load

9.  Attach a DC supply to the circuit input (VI) with the supply voltage set to 0 V.  Slowly turn up the supply to 6 V.  Note what happens to the LED.

10.  Remove the LED and insert the op-amp circuit as shown below.  Set the DC input to 0 V.  Measure the open-circuit voltage of the op-amp output as you slowly turn up the DC input from 0 V to 6 V.

10k

+

-

vI

High Output Resistance Source

+

--

+15

-15

3

2

6

4

7

0.620k

+

-

vO

Buffer Circuit

Q8:  Based on this measurement, what is the apparent open-circuit voltage gain of this Op-Amp circuit?

12.  Turn the DC input down to 0 V.  Attach the LED as shown below.  Record what happens as the input is turned up to 6 V.

10k

+

-

vI

High Output Resistance Source

+

--

+15

-15

3

2

6

4

7

0.620k

+

-

vO

Buffer Circuit

LED

+

-

Low Resistance Load

Q9:  Why was the LED brighter this time despite the same DC input voltage?