Special Problem 1.5-2

Amplifier A1 has:

an input resistance of 750 .

an output resistance of 10K.

an open-circuit voltage gain of 100 V/V.

Amplifier A2 has:

an input resistance of 30 K.

an output resistance of 50 .

an open circuit voltage gain of 1 V/V.

A voltage source with a source resistance of 250 is connected to the input of the common-emitter amp.

The output of the first amp (A1) is connected to the input of the second (A2).

The output of the second amp is connected to a load of 50 .

vS

50

250

+

vO

_

+

-

A1

A2

I.E.,

What is the value of vo/vs ?