Special Problem 5.6-4

In the circuit below, Q1 has K=1.0 mA/V2 and Vt1 = 1.0 V.

The transistor Q2 likewise has K = 1 mA/V2, but has a threshold voltage of Vt2 = 2.0 V.

In other words Q1 and Q2 are not identical!

The resistor R2 has been selected such that Q2 is in saturation.

1) Determine R1 (note I said R1!)so that the drain current of Q2 (note I said Q2!) is 4.0 mA.

2) What is the largest possible value of resistor R2 so that Q2 remains in saturation?

R1

Q2

13.0V

Q1

K=1 mA/V2

Vt2 = 2.0 V

13.0V

R2

4.0 mA

K=1 mA/V2

Vt1 = 1.0 V