5.2 Current-Voltage Characteristics (of BJTs)

We will find that BJTs are in many was similar and analogous to MOSFETs!

For example, we will find:

BJT							MOSFET

Base          is analogous to        Gate

Collector		“			 Drain

Emitter			“			Source

Cutoff			“			Cutoff

Saturation		“			Triode

Active			“			Saturation

iC				“			iD

vBE				“			vGS

vDS				“			vCE

Look for these analogies to help you understand BJTs!

One important difference between BJTs and MOSFETs is that:

iB not equal to zero !!!

Therefore iE not equal to iC  !!!