Example: A BJT Circuit in Saturation

10.0 K

2.0 K

5.7 V

10 K

10.7 V

 = 99

Determine all currents for the BJT in the circuit below.

Hey! I remember this circuit, its just like a previous example.  The BJT is in active mode!

Let’s see if you are correct!  ASSUME it is in active mode and ENFORCE VCE = 0.7 V and  iC = iB.

The B-E KVL is therefore:

5.7 – 10 iB –0.7 – 2 (99+1) iB=0

Therefore iB = 23.8 A

See! Base current iB = 23.8 A, just like before.  Therefore collector current and emitter current are again iC = 99iB = 2.356 mA and iE = 100 iB = 2.380 mA.  Right ?!

Well maybe, but we still need to CHECK to see if our assumption is correct!

We know that iB = 23.8 A > 0  , but what about VCE ?

From collector-emitter KVL we get:

10.7 – 10 iC – VCE – 2 iE =0

Therefore,

VCE = 10.7 – 10(2.36) – 2(2.38) = -17.66 V < 0.7 V  X

Our assumption is wrong ! The BJT is not in active mode.

In the previous example, the collector resistor was 1K , whereas in this example the collector resistor is 10K.  Thus, there is 10X the voltage drop across the collector resistor, which lowers the collector voltage so much that the BJT cannot remain in the active mode.

Q:  So what do we do now ?

A:  Go to Step 5; change the assumption and try it again!

Lets ASSUME instead that the BJT is in saturation.  Thus, we ENFORCE the conditions:

VCE = 0.2  V     VBE = 0.7 V    VCB = -0.5 V

Now lets ANALYZE the circuit !

10.0 K

2.0 K

5.7 V

10 K

10.7 V

+

0.2

-

+

0.7

-

+

-0.5

-

iB

iE

iC

Note that we cannot directly determine the currents, as we do not know the base voltage, emitter voltage, or collector voltage.

But, we do know the differences in these voltages!

For example, we know that the collector voltage is 0.2 V higher than the emitter voltage, but we do not know what the collector or emitter voltages are!

Q: So, how the heck do we ANALYZE this circuit !?

A: Often, circuits with BJTs in saturation are somewhat more difficult to ANALYZE than circuits with active BJTs.  There are often many approaches, but all result from a logical, systematic application of Kirchoff’s Laws!

ANALYSIS EXAMPLE 1 – Start with KCL

We know that  iB + iC = iE  (KCL)

But, what are iB, iC, and iE ??

Well, from Ohm’s Law:

Therefore, combining with KCL:

Look what we have, 1 equation and 3 unknowns.

We need 2 more independent equations involving VB, VC, and VE!

Q:  Two more independent equations !? It looks to me as if we have written all that we can about the circuit using Kirchoff’s Laws.

A: True!  There are no more independent circuit equations that we can write using KVL or KCL !  But, recall the hint sheet:

“Make sure you are using all available information”.

There is more information available to us – the ENFORCED conditions!

VCE = VC – VE = 0.2                VC = VE + 0.2

VBE = VB – VE = 0.7                VB = VE + 0.7

Two more independent equations! Combining with the earlier equation:

One equation and one unknown !  Solving, we get VE =2.2 V.

Inserting this answer into the above equations, we get:

VB = 2.9 V     VC = 2.4 V

iC = 0.83 mA        iB = 0.28 mA       iE = 1.11 mA

ANALYSIS EXAMPLE 2 – Start with KVL

10.0 K

2.0 K

5.7 V

10 K

10.7 V

+

0.2

-

+

0.7

-

+

-0.5

-

iB

iE

iC

We can write the KVL equation for any two circuit legs:

B-E KVL:

5.7 – 10 iB – 0.7 – 2 iE = 0.0

C-E KVL:

10.7 – 10 iC – 0.2 – 2 iE = 0.0

Note the ENFORCED conditions are included in these KVL equations.

Simplifying, we get these 2 equations with 3 unknowns:

5.0  = 10 iB  + 2 iE

10.5 = 10 iC + 2 iE

We need one more independent equation involving iB, iC, and iE.

Try KCL !                         iB + iC = iE

Inserting the KCL equation into the 2 KVL equations, we get:

5.0  = 12 iB  + 2 iC

10.5 = 2 iB + 12 iC

Solving, we get the same answers as in analysis example 1.

Lesson:  There are multiple strategies for analyzing these circuits; use the ones that you feel most comfortable with !

However you ANALYZE the circuit, you must in the end also CHECK your results.

First CHECK to see that all currents are positive:

iC = 0.83 mA > 0     iB = 0.28 mA > 0     iE = 1.11 mA > 0 

Also CHECK collector current:

iC = 0.83 mA <  iB = 27.7 mA  

Our solution is correct !!!