Example: Another DC Analysis of a BJT Circuit

Find the collector voltages of the two BJTs in the circuit below.

5.3 V

10.0 V

7.7 V

50 K

1 K

1.0 K

1 K

Q2

Q1

iC1

iC2

i1

iB2

 = 100

 = 100

ASSUME both BJTs are in active mode, therefore ENFORCE

Q: Now, how do we ANALYZE the circuit ??

A:  This seems to be a problem ! We cannot easily solve the emitter base KVL, as i1 is NOT EQUAL to iE1 (make sure you understand this !).   Instead, we find:

So, what do we do ?

First, ask the question:  What do we know ??

Look closely at the circuit, it is apparent that VB1 = 5.3 V and VE2 = 7.7 V.

5.3 V

10.0 V

7.7 V

50 K

1 K

1.0 K

1 K

Q2

Q1

iC1

iC2

i1

iB2

 = 100

 = 100

Hey! We therefore also know VE1 and VB2:

Wow ! From these values we get:

and

This is easy!  Since we know i1 and iB2, we can find iE1:

iE1 =i1 + iB2 = 4.0 +0.02 = 4.02 mA

Since we know one current for each BJT, we know all currents for each BJT:

Finally, we can determine the voltages VC1 and VC2.

VC1 = 0.0 + 1 iC1 = 0.0 + 1(3.98) = 3.98 V

VC2 = 0.0 + 1 iC2 = 0.0 + 1(2.0) = 2.0 V

Now, let’s CHECK to see if our assumptions were correct:

iC2 = 2mA > 0                    iC1 = 3.98 mA > 0

Assumptions are correct !